MATERIAL SAFETY DATA SHEET

Spectrum Ethers Pvt. Ltd.

PROPICONAZOLE TECHNICAL

Rasegaon Village, Nashik (M.S.)

Contry: India, Pin-422202

Emergency Phone: 02557 – 228000

| 1. Product and Company Identification |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Name | : | Propiconazole Technical |  |  |  |  |  |  |  |  |
| Chemical Name | : | (2RS,4RS;2RS,4SR)-1-[2-(2,4-dichlorophenyl)-4-propyl-1,3-dioxolan- |  |  |  |  |  |  |  |  |
|  |  | 2-ylmethyl]-1H-1,2,4-triazole |  |  |  |  |  |  |  |  |
| Chemical Class | : | Triazole fungicides |  |  |  |  |  |  |  |  |
| Molecular formula | : | C15H17Cl2N3O2 |  |  |  |  |  |  |  |  |
| 2. Composition / Information on ingredients |  |  |  |  |  |  |  |  |  |  |
| Chemical Name |  |  | CAS# |  |  |  |  |  |  | Percent or content (w/w) |
| Propiconazole |  |  | 60207-90-1 |  |  |  |  |  |  | 92.00 % |
| Other ingredients |  |  | - |  |  |  |  |  |  | 8.00% |
| 3. Health Hazards Identification |  |  |  |  |  |  |  |  |  |  |
| Toxicity | : This product is slightly hazardous |  |  |  |  |  |  |  |  |  |
| Inhalation | : | Slightly toxic |  |  |  |  |  |  |  |  |
| Ingestion | : | Slightly toxic |  |  |  |  |  |  |  |  |
| Skin | : | Mild irritant - skin sensitizer |  |  |  |  |  |  |  |  |
| Eye | : | Mild irritant |  |  |  |  |  |  |  |  |
| Sensitization | : | Non-sensitizer to skin |  |  |  |  |  |  |  |  |
| 4. First Aid Measures |  |  |  |  |  |  |  |  |  |  |
| Eyes | Hold eye open and rinse gently with fresh water for 15-20 minutes. If |  |  |  |  |  |  |  |  |  |
|  | Irritation persists, consult a medical Physician. |  |  |  |  |  |  |  |  |  |
| Skin | Take off contaminated clothing. Wash skin immediately with plenty of |  |  |  |  |  |  |  |  |  |
|  | Sop & Water for 15-20 minutes. |  |  |  |  |  |  |  |  |  |
| Ingestion | Give a large quantity of medical charcoal in plenty of water. Seek medical |  |  |  |  |  |  |  |  |  |
|  | attention if a large column of concentrate was ingested. |  |  |  |  |  |  |  |  |  |

Inhalation	Move person to fresh air. Give artificial respiration if required and call ambulance. Seek immediate medical assistance.

Page 1 of 6

5. Fire Fighting Measures

| Fire |  | Not expected to be a fire hazard |  |  |
| --- | --- | --- | --- | --- |
| Explosion |  | None identified |  |  |
| Suitable Extinguishing |  | Use water fog, foam, CO2 to extinguish flames. |  |  |
| Media |  |  |  |  |
| Flammability Properties Non-flammable, product is combustible |  |  |  |  |
| 6. Accidental Release Measures |  |  |  |  |
| Personal precautions | : | Wear suitable protective clothing, shoes, gloves and goggle. |  |  |
| In case of spill or leak | : | Control the spill at its source. Control the spill to prevent it from |  |  |
|  |  | spreading, contaminating soil, or entering sewage and drainage |  |  |
|  |  | systems or any body of water. |  |  |
|  |  | Clean the spills immediately. Cover the entire spill with absorbing |  |  |
|  |  | material and place into compatible disposal container. Scrub area |  |  |
|  |  | with hard water detergent. Pickup wash liquid with additional |  |  |
|  |  | absorbent and place into compatible disposal container. Once all |  |  |
|  |  | material is cleaned up and placed in disposal containers, seal |  |  |
|  |  | container and arrange for disposition. |  |  |
| 7. Handling and Storage |  |  |  |  |
| Handling | : | Prevent eating, drinking, tobacco use and cosmetic application in |  |  |
|  |  | areas. Where there is a potential for exposure to the material. |  |  |
|  |  | Wash thoroughly with soap and water after handling. |  |  |
| Storage | : | Keep tightly closed in original labeled container. Keep away from |  |  |
|  |  | the reach of children. Keep away from foods, drinks and animals |  |  |
|  |  | feeding stuffs. |  |  |
| Suitable materials & | : | Aluminum and Stainless steel. |  |  |
| Coatings |  |  |  |  |

Page 2 of 6

| Loading & uploading | : | Ambient |  |
| --- | --- | --- | --- |
| Temperature |  |  |  |
| Transport |  | Ambient |  |
| Temperature |  |  |  |
| Specific use |  | Use this product only for plant protection under outdoor |  |
|  |  | conditions. |  |
| 8. Exposure Controls/Personal Protection |  |  |  |
| Ingestion | Prevent eating, drinking, tobacco use, and cosmetic application in areas |  |  |
|  | where there is a potential for exposure to the material. |  |  |
| Eye protection | Where eye contact is likely, use chemical splash goggles. Facilities storing |  |  |
|  | or utilizing this material should be equipped with an eyewash facility and |  |  |
|  | a safety shower. |  |  |
| Skin Protection | Where contact is likely wear chemical- resistant gloves, coverall, socks |  |  |
|  | And chemical-resistant foot wear. For over head exposure, wear |  |  |
|  | chemical-resistant head gear. |  |  |
| Inhalation | Use approved chemical cartridge respirator with organic vapor cartridges |  |  |
|  | and Pre- filters or a supplied air respirator. |  |  |
| 9. Physical and Chemical Properties |  |  |  |
| Appearance |  | Transparent amber colour liquid |  |
| Odour |  | Musty odour |  |
| Melting Point |  | 23°C (glass transition temperature) |  |
| Boiling Point |  | 180 °C (356 °F; 453 K) at 0.1 mmHg [1] |  |
| pH |  | 5.77 |  |
| Vapour Pressure |  | 6.7 x10-5Pa at 25°C |  |
| Water solubility |  | 100 mg/l (20oC) |  |
| Relative density |  | 1.282 (20oC) |  |
| Flash Point |  | 233oC |  |

Page 3 of 6

10. Stability and Reactivity

| Chemical Stability |  |  | stable material |  |  |
| --- | --- | --- | --- | --- | --- |
| Hazardous Polymerization |  |  | Will not occur |  |  |
| Incompatibilities |  |  | Not known |  |  |
| 11. Toxicological Information |  |  |  |  |  |
| Acute Oral Toxicity |  |  | LD50 (Rats) 300-2000 mg/kg |  |  |
| Acute Dermal Toxicity |  |  | LD50 (Rats) 2,000 mg/kg b.w |  |  |
| Acute Inhalation |  |  | LC50 (rats): 5.13 mg / L air (4 h) |  |  |
| Eye Irritation |  |  | Rabbit: Mild irritant |  |  |
| Skin Irritant |  |  | Rabbit: Mild irritant |  |  |
| Skin Sensitization |  |  | Guinea pig: Sensitizing |  |  |
| 12. Ecological Information |  |  |  |  |  |
| Birds | Acute oral LD50 (Japanese quail) 2223 mg/kg bw |  |  |  |  |
|  |  | LC50 (5 d, Japanese quail) >10,000 mg/kg diet |  |  |  |
|  | Acute oral LD50 (mallard ducks) >2510 mg/kg bw |  |  |  |  |
|  |  | LC50 (5 d, mallard ducks) >5620 mg/kg diet |  |  |  |
| Fish | LC50 (96 h) for Rainbow trout 4.3 mg/l, |  |  |  |  |
|  |  |  | Carp 6.8 mg/l |  |  |
| Bees | LD50 (oral & contact) >100 g/bee |  |  |  |  |
|  | LD50 (contact) >200 g/bee |  |  |  |  |
| 13. Disposal Considerations |  |  |  |  |  |
| Waste | Disposal | Do not reuse product containers. Dispose of product containers, waste |  |  |  |
| Method |  | containers and residues according to local, state and federal health and |  |  |  |
|  |  | environmental regulations. |  |  |  |

Page 4 of 6

14. Transport Information

Rail/Road (RID/ADR)

| Proper Shipping Name | : | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S |  |  |
| --- | --- | --- | --- | --- |
| Class | : | 9 |  |  |
| UN No. | : | 3082 |  |  |
| Packing Group | : | III |  |  |
| Sea (IMDG – Code) |  |  |  |  |
| Proper Shipping Name | : | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S |  |  |
| Class | : | 9 |  |  |
| IMDG No. | : | 3082 |  |  |
| Packing Group | : | III |  |  |
| Marine Pollutant | : | Yes |  |  |
| Air (ICAO/IATA) |  |  |  |  |
| Proper Shipping Name | : | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S |  |  |
| Class | : | 9 |  |  |
| UN No. | : | 3082 |  |  |
| Packing Group | : | III |  |  |
| 15. Regulatory Information |  |  |  |  |
| EU- Classification Hazard |  | Xn | Harmful |  |
| Symbol |  | N | Dangerous for the environment |  |
| Special Risks (R-Phrases) |  | 22 | Harmful if swallowed |  |
|  |  | 43 | May cause sensitization by skin contact |  |
|  |  | 50/53 | Very toxic to aquatic organisms, may cause long-term |  |
|  |  |  | adverse effects in the aquatic environment |  |
| Safety Advice (S- Phrases) |  | 36/37 | Wear suitable protective clothing and gloves. |  |
|  |  | 46 | If swallowed, seek medical advice immediately and |  |
|  |  |  | show this container or label. |  |
|  |  | 60 | This material and its container must be disposed of as |  |
|  |  |  | hazardous waste |  |
|  |  | 61 | Avoid release to the environment. Refer to special |  |
|  |  |  | instructions/Safety data sheets. |  |

Page 5 of 6

International WHO Toxicity	III	Slightly hazardous

Classification

16. Other Information

The information above is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we assume no liability resulting from its use. Users should make their own investigations to determine the suitability of the information for their particular purposes. In no event shall we be liable for any claims, losses, or damages of any third party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if we have been advised of the possibility of such damages.

Page 6 of 6